# Step 1: Align the robot to the hallway

Check to see if the robot is facing a wall

##(Front is a solid color, and the boundaries for the floor and/or ceiling are parallel), returning a tangent value to determine

If not aligned

Check which wall is closer (Color will be more prominent in camera view)

Rotate in that direction by 10 degrees toward the more prominent wall

Repeat step 1

If Aligned

Move forward until the rover makes contact with the wall

Move backward until it makes contact with the other wall

Record the distance traveled between walls

Move forward by distance / 2 (So the rover is in the center of the hall)

Turn 90 degrees right

# Step 2: Move to an intersection

Check for the intersection light in the camera view

If color shows up

Calculate distance, perhaps through trigonometry

If the intersection light is not seen, or is greater than 1 unit away

Move one unit forward

Else if the intersection is one unit away

Move one unit forward

Declare that you have reached the intersection to terminate the navigation loop

# Step 3: Check hallways for the goal

## Do a full rotation to check all possible intersections

## Before returning to the original rotation

If not seen goal and total rotation < 360

Rotate 90 degrees to the right

Check for the goal color

If seen goal

Move forward 1 unit

Repeat until the contact sensor goes off, meaning you made contact with the goal

Else

Turn right 90 degrees

If there’s a wall instead of a hallway

Turn 90 degrees left

Start over from step 2

## This sequence lets the rover explore all hallways using the right-hand rule (aka: recursive backtracking)